Table of Contents

# JTAG Enumeration for Security Research on the Zynq UltraScale+ MPSoC

## 1. Introduction

### 1.1 Why JTAG enumeration matters

JTAG is the most privileged hardware-level debug interface on a modern SoC. On an unhardened device, it provides:

- Read/write access to every memory location the SoC can address

- Halt and step control of every CPU (APU, RPU, PMU)

- Direct access to the security control unit’s registers

- Read access to eFUSE shadow registers including device fingerprint, secure-boot policy, and PPK hashes

- The ability to release CPUs from reset and execute arbitrary code at EL3 — the highest ARMv8 privilege level, above any OS, hypervisor, or TrustZone monitor

For an attacker with physical access, JTAG bypasses the operating system entirely. For a defender, JTAG state is what the silicon actually enforces — software-level hardening is irrelevant if the JTAG hardware gates are wide open.

The first question on any new device is: what is the silicon actually gating, and what is it leaving open? Answering it requires reading and decoding roughly eighty registers across seventeen functional sections — the security spine plus power, clocks, resets, A53 state, code-execution discovery, debug topology, memory map probes, peripheral and memory protection units, RPU configuration, IPC fabric, and FPGA configuration status — plus tracking the live state of every domain that could be a debug-relevant target. Doing this by hand is slow and error-prone. The enumeration tool described in Volumes 2 and 3 does it in ten seconds and produces a structured, diffable report.

## 2. Background

AMD’s Zynq UltraScale+ MPSoC is the platform every step of this workflow targets. The subsections below cover its high-level architecture, variant landscape, boot model, JTAG security gates, and memory layout — in the order needed to understand what enumeration produces.

### 2.1 The ZynqMP SoC at a glance

The Zynq UltraScale+ MPSoC is a multi-die system-on-chip combining several processor subsystems with FPGA fabric:

ZynqMP subsystem overview showing LPD, FPD, and PL domains

Two power domains govern everything:

- LPD — Low-Power Domain. Always-on. Contains the RPU, PMU, CSU, OCM (on-chip SRAM), most peripherals

- FPD — Full-Power Domain. Can be gated off. Contains the APU, DDR controller, GPU, VCU, PCIe, SATA, USB3, DisplayPort, GIC

A device booted “JTAG-idle” with the FPD off can still service JTAG debug because the LPD remains powered.

### 2.2 Boot model

The ZynqMP boot sequence is multi-stage:

ZynqMP boot flow from PS_POR_B through Linux kernel load

The JTAG-idle state is the cleanest baseline for enumeration: APUs are in reset, no FSBL has run, no secrets have been loaded into peripherals, no peripheral interrupts are active. Most of this paper’s sample data comes from a ZCU102 in JTAG idle.

A device booted from SD/QSPI/eMMC produces a much richer enumeration report because FSBL has run, DDR is initialized, PMU firmware is running, peripherals are alive, and U-Boot or Linux may be visible in memory. The two report types diff against each other to reveal what the FSBL chain actually did.

### 2.3 JTAG security architecture

This is the part that drives most of the rest of the series.

#### JTAG chain composition

The ZynqMP exposes two TAPs in series on the JTAG chain:

JTAG chain composition: PS-TAP + ARM CoreSight DAP with Access Ports

The PS-TAP is Xilinx-proprietary; the ARM DAP is the standard CoreSight ARM debug-access port. Both must be reachable for full enumeration.

#### The CSU Secure Stream Switch (SSS)

The CSU implements a multi-input multi-output stream switch that gates which JTAG paths can talk to which security-sensitive blocks. Three SSS paths matter for JTAG enumeration:

JTAG ── ┬─► SSS_DAP_SEC ──► ARM DAP (APU, RPU debug)
        │
        ├─► SSS_PLTAP_SEC ──► PL TAP (FPGA fabric JTAG)
        │
        └─► SSS_PMU_SEC ──► PMU (firmware execution control)

Each path’s gate is controlled by a 3-bit magic value in CSU.JTAG_SEC (at 0xFFCA0038). Open requires 0b111; any other value gates the path. The 3-bit design is anti-glitch — a single bit flip from fault injection cannot unlock a gated path.

#### Per-cluster debug authorization (JTAG_DAP_CFG)

A second register CSU.JTAG_DAP_CFG (at 0xFFCA003C) controls what the ARM DAP itself can do, with six 1-bit gates:

| Bit | Field | Effect |
| --- | --- | --- |
| 0 | SSSS_APU_DBGEN | APU halt/step/breakpoint allowed (non-secure world) |
| 1 | SSSS_APU_NIDEN | APU non-invasive trace (ETM, etc.) allowed |
| 2 | SSSS_APU_SPIDEN | APU SECURE-world debug allowed (EL3/TrustZone) |
| 3 | SSSS_APU_SPNIDEN | APU secure-world trace allowed |
| 4 | SSSS_RPU_DBGEN | RPU (R5) debug allowed |
| 5 | SSSS_RPU_NIDEN | RPU trace allowed |

These are cluster-wide, not per-core. SPIDEN is the bit that defines whether the device’s secure-world is attackable via JTAG.

#### eFUSE secure-boot policy

Finally, the one-way fuses in EFUSE.SEC_CTRL (at 0xFFCC1058) control the silicon’s secure-boot policy at hardware level:

| Bit(s) | Field | Effect when set |
| --- | --- | --- |
| 0 | AES_RDLK | AES key in eFUSE cannot be read |
| 1 | AES_WRLK | AES key in eFUSE cannot be written |
| 2 | ENC_ONLY | Boot image MUST be encrypted |
| 3 | BBRAM_DIS | BBRAM disabled as key source |
| 5 | JTAG_DIS | Hardware JTAG disable — irreversible |
| 10 | SEC_LOCK | Lock SEC_CTRL itself — no further fuses settable |
| [25:11] | RSA_EN | 15-bit magic field requiring specific value to enforce RSA-signed boot |
| 26 | PPK0_WRLK | Primary Public Key 0 hash write-locked |
| [28:27] | PPK0_INVLD | 2-bit magic field; invalidates PPK0 |
| 29 | PPK1_WRLK | PPK1 hash write-locked |
| [31:30] | PPK1_INVLD | 2-bit magic field; invalidates PPK1 |

Two fields use magic-value patterns (RSA_EN, PPK_INVLD) rather than single-bit enables — the same anti-glitch defense as the JTAG_SEC gates. A successful glitch attack on a single bit cannot enable RSA bypass or invalidate a PPK.

These three register groups — JTAG_SEC, JTAG_DAP_CFG, and EFUSE.SEC_CTRL — define essentially the entire JTAG-visible security policy of the device. Reading them is the heart of the enumeration workflow described in Volume 2.

### 2.4 Memory layout overview

ZynqMP’s address space (identical across MPSoC variants):

| Range | Block | Notes |
| --- | --- | --- |
| 0x00000000 - 0x7FFFFFFF | DDR (lower 2 GB) | Main system DRAM. Only accessible after DDR controller init by FSBL. |
| 0xC0000000 - 0xFCFFFFFF | PL aperture / PCIe | Programmable Logic and PCIe BARs |
| 0xFD000000 - 0xFD0FFFFF | DDR controller |  |
| 0xFD1A0000 - 0xFD1AFFFF | CRF_APB | FPD Clock/Reset (APLL/DPLL/VPLL, RST_FPD_*) |
| 0xFD400000 - 0xFD4FFFFF | SERDES | Gigabit transceivers |
| 0xFD5C0000 - 0xFD5CFFFF | APU registers | RVBARADDR0-3 reset vectors. APU power/reset. |
| 0xFD800000 - 0xFD9FFFFF | GPU (Mali-400) |  |
| 0xFE000000 - 0xFE00FFFF | GIC distributor |  |
| 0xFE100000 - 0xFE1FFFFF | VCU (video codec) | EV variants only |
| 0xFF000000 - 0xFF0FFFFF | PS UARTs, I2C, SPI, GPIO, QSPI |  |
| 0xFF160000 - 0xFF17FFFF | SDIO0/1 |  |
| 0xFF300000 - 0xFF33FFFF | IPI mailboxes | PMU↔APU↔RPU communication |
| 0xFF410000 - 0xFF41FFFF | LPD_SLCR |  |
| 0xFF5E0000 - 0xFF5EFFFF | CRL_APB | LPD Clock/Reset (IOPLL/RPLL, BOOT_MODE, RST_LPD_*) |
| 0xFF9A0000 - 0xFF9AFFFF | RPU configuration |  |
| 0xFFC80000 - 0xFFCBFFFF | CSUDMA + CSU | CSU at 0xFFCA0000 |
| 0xFFCC0000 - 0xFFCCFFFF | eFUSE controller | DNA, SEC_CTRL, PPK hashes |
| 0xFFCD0000 - 0xFFCDFFFF | BBRAM | Battery-Backed RAM (AES key storage) |
| 0xFFCE0000 - 0xFFCEFFFF | RSA core | Hardware RSA-2048/4096 |
| 0xFFD80000 - 0xFFD8FFFF | PMU_GLOBAL | Power/reset of every domain |
| 0xFFDC0000 - 0xFFDDFFFF | PMU RAM (LMB) | PMU firmware runs here |
| 0xFFE00000 - 0xFFE3FFFF | RPU TCM | Tightly-Coupled Memory for R5 cores |
| 0xFFFC0000 - 0xFFFFFFFF | OCM (4×64 KB SRAM) | Default FSBL/ATF load region |

This layout is identical across ZCU102/104/106, Ultra96, custom ZynqMP boards, and RFSoCs — the variant-specific differences (VCU, RF tiles, GPU presence) are bonded selections within these address ranges, not different memory maps.